/*

SparkFun Inventor's Kit

Circuit02Expansion

Adapted from Oomlout's SIK Circuit 2

*/

//LED Pin Variables

int ledPins[] = {2,3,4,5,6,7,8,9}; 		//An array to hold the pin each LED is connected to

//i.e. LED #0 is connected to pin 2, LED #1, 3 and so on

//to address an array use ledPins[0] this would equal 2

//and ledPins[7] would equal 9

//PWM values for analogWrite on pin #9, ERASE COMMENT INDICATORS TO USE, switch lines for different questions

// you may have trouble getting your motor running with lower PWM values.

int PWMvalue = 0;		//0% power

// int PWMvalue = 63.75;		//25% power

// int PWMvalue = 127.5;		//50% power

// int PWMvalue = 191.25;	//75% power

// int PWMvalue = 255;		//100% power

/*

* setup() - this function runs once when you turn your Arduino on

* We set the three control pins to outputs

*/

void setup()

{

//Set each pin connected to an LED to output mode (pulling high (on) or low (off)

for(int i = 0; i < 8; i++){        			 //this is a loop and will repeat eight times

pinMode(ledPins[i],OUTPUT); 		//we use this to set each LED pin to output

}                                 				//the code this replaces is below

/* (commented code will not run)

* these are the lines replaced by the for loop above they do exactly the

* same thing the one above just uses less typing

pinMode(ledPins[0],OUTPUT);

pinMode(ledPins[1],OUTPUT);

pinMode(ledPins[2],OUTPUT);

pinMode(ledPins[3],OUTPUT);

pinMode(ledPins[4],OUTPUT);

pinMode(ledPins[5],OUTPUT);

pinMode(ledPins[6],OUTPUT);

pinMode(ledPins[7],OUTPUT);

(end of commented code)*/

}

void loop(){

analogWrite (9, PWMvalue);

}