/*

SparkFun Inventor's Kit

Circuit10BExpansion

Adapted from Arduino Experimentation Kit Example Code

Circ05 8 more LEDs (74HC595 Shift Register)

&

CIRC-10 .: Temperature :. (TMP36 Temperature Sensor)

*/

//Pin Definitions

//The 74HC595 using a protocol called SPI (for more details http://www.arduino.cc/en/Tutorial/ShiftOut)

//Which has three pins

int data = 2;

int clock = 3;

int latch = 4;

int temperaturePin = 0;

/*

* setup() - this function runs once when you turn your Arduino on

* We set the three control pins to outputs

*/

void setup()

{

Serial.begin(9600);

pinMode(data, OUTPUT);

pinMode(clock, OUTPUT);

pinMode(latch, OUTPUT);

}

void loop()                     // run over and over again

{

float temperature = getVoltage(temperaturePin);  //get voltage from sensor

temperature = (temperature - .5) * 100;           //Celsius

//convert from 10 mv per ˚ with 500 mV offset to ˚((volts - 500mV) times 100)

//a bunch of if statements to light # of LEDs depending

//on temperature, calls tempShift method, passes LEDs on/off in binary

if(temperature <= 15){

tempShift(0b00000000);

}

if(temperature <= 17.5 && temperature > 15){

tempShift(0b10000000);

}

if(temperature <= 20 && temperature > 17.5){

tempShift(0b11000000);

}

if(temperature <= 22.5 && temperature > 20){

tempShift(0b11100000);

}

if(temperature <= 25 && temperature > 22.5){

tempShift(0b11110000);

}

if(temperature <= 27.5 && temperature > 25){

tempShift(0b11111000);

}

if(temperature <= 30 && temperature > 27.5){

tempShift(0b11111100);

}

if(temperature <= 32.5 && temperature > 30){

tempShift(0b11111110);

}

if(temperature > 32.5){

tempShift(0b11111111);

}

Serial.println(temperature);

}

float getVoltage(int pin){

return (analogRead(pin) * .004882814);

//converting from a 0 to 1024 digital range

// to 0 to 5 volts (each 1 reading equals ~ 5 millivolts

}

void tempShift(int j){

digitalWrite(latch, LOW);     //Pulls the chips latch low

shiftOut(data, clock, MSBFIRST, j); //Shift j to shift register

digitalWrite(latch, HIGH);   //Pulls the latch high displaying the data

}