/*

SparkFun Inventor's Kit

Circuit10Expansion

Adapted from Oomlout's SIK Circuit 10

*/

int temperaturePin = 0; 	// declare the pin the temp sensor is connected to

int LEDpin = 9;		// declare the pin the LED is connected to

// setup() - this function runs once when you turn your Arduino on

void setup()

{

Serial.begin(9600);  		// Start the serial connection with the computer

// to view the result open the serial monitor

// last button beneath the file bar (looks like a box with an antennae)

}

void loop()                     // loop runs over and over again while Arduino has power

{

float temperature = getVoltage(temperaturePin);

// line above gets the voltage reading from the temperature sensor

temperature = (temperature - .5) * 100;

// line above converts from 10 mV per degree with 500 mV offset to degrees in Celsius

int tempBright = map(temperature, 20, 30, 0, 255);

// maps temperature to an analogWrite (PWM) friendly value (0-255)

// you may have to change the above numbers 20 & 30 depending on how warm or cold it is.

Serial.println(temperature);          //prints the results to Serial

analogWrite(LEDpin, tempBright);

// line above uses analogWrite to change brightness of LED to tempBright

delay(1000);       // waits 1 second

}

// getVoltage() - returns a voltage from the original digital temp sensor value

float getVoltage(int pin){

return (analogRead(pin) * .004882814);

//line above converts from a 0 to 1024 digital range to 0 to 5 volts (each 1 digital equals ~ 5 millivolts)

}