/*

RGB_LED_Color_Fade_Cycle.pde

Cycles through the colors of a RGB LED

Written for SparkFun Arduino Inventor's Kit CIRC-RGB

*/

// LED leads connected to PWM pins

const int RED_LED_PIN = 9;

const int GREEN_LED_PIN = 10;

const int BLUE_LED_PIN = 11;

// Used to store the current intensity level of the individual LEDs

int redIntensity = 0;

int greenIntensity = 0;

int blueIntensity = 0;

// Length of time we spend showing each color

const int DISPLAY_TIME = 500; // In milliseconds

void setup() {

pinMode(speakerPin, OUTPUT);

}

void loop() {

// Cycle color from red through to green

// (In this loop we move from 100% red, 0% green to 0% red, 100% green)

for (greenIntensity = 0; greenIntensity <= 255; greenIntensity+=5) {

redIntensity = 255-greenIntensity;

analogWrite(GREEN_LED_PIN, greenIntensity);

analogWrite(RED_LED_PIN, redIntensity);

int tone = map(greenIntensity, 0, 255, 956, 1276 ); //maps RGB color to low tones

playTone(tone, DISPLAY_TIME);                       //calls method that plays tone

}

// Cycle color from green through to blue

// (In this loop we move from 100% green, 0% blue to 0% green, 100% blue)

for (blueIntensity = 0; blueIntensity <= 255; blueIntensity+=5) {

greenIntensity = 255-blueIntensity;

analogWrite(BLUE_LED_PIN, blueIntensity);

analogWrite(GREEN_LED_PIN, greenIntensity);

int tone = map(blueIntensity, 0, 255, 1277, 1595 ); //maps RGB color to mid tones

playTone(tone, DISPLAY_TIME);                       //calls method that plays tone

}

// Cycle cycle from blue through to red

// (In this loop we move from 100% blue, 0% red to 0% blue, 100% red)

for (redIntensity = 0; redIntensity <= 255; redIntensity+=5) {

blueIntensity = 255-redIntensity;

analogWrite(RED_LED_PIN, redIntensity);

analogWrite(BLUE_LED_PIN, blueIntensity);

int tone = map(redIntensity, 0, 255, 1596, 1915 ); //maps RGB color to high tones

playTone(tone, DISPLAY_TIME);                       //calls method that plays tone

}

}

void playTone(int tone, int duration) { // method that plays a tone or note

for (long i = 0; i < duration * 1000L; i += tone * 2) {

digitalWrite(speakerPin, HIGH);

delayMicroseconds(tone);

digitalWrite(speakerPin, LOW);

delayMicroseconds(tone);

}

}