/*

RGB_LED_Color_Fade_Cycle.pde

Cycles through the colors of a RGB LED

Written for SparkFun Arduino Inventor's Kit CIRC-RGB

*/

// LED leads connected to PWM pins

const int RED_LED_PIN = 9;

const int GREEN_LED_PIN = 10;

const int BLUE_LED_PIN = 11;

// Used to store the current intensity level of the individual LEDs

int redIntensity = 0;

int greenIntensity = 0;

int blueIntensity = 0;

// Length of time we spend showing each color

const int DISPLAY_TIME = 100; // In milliseconds

void setup() {

// No setup required.

}

void loop() {

// Cycle color from red through to green

// (In this loop we move from 100% red, 0% green to 0% red, 100% green)

for (greenIntensity = 0; greenIntensity <= 255; greenIntensity+=5) {

redIntensity = 255-greenIntensity;

analogWrite(GREEN_LED_PIN, greenIntensity);

analogWrite(RED_LED_PIN, redIntensity);

delay(DISPLAY_TIME);

}

// Cycle color from green through to blue

// (In this loop we move from 100% green, 0% blue to 0% green, 100% blue)

for (blueIntensity = 0; blueIntensity <= 255; blueIntensity+=5) {

greenIntensity = 255-blueIntensity;

analogWrite(BLUE_LED_PIN, blueIntensity);

analogWrite(GREEN_LED_PIN, greenIntensity);

delay(DISPLAY_TIME);

}

// Cycle cycle from blue through to red

// (In this loop we move from 100% blue, 0% red to 0% blue, 100% red)

for (redIntensity = 0; redIntensity <= 255; redIntensity+=5) {

blueIntensity = 255-redIntensity;

analogWrite(RED_LED_PIN, redIntensity);

analogWrite(BLUE_LED_PIN, blueIntensity);

delay(DISPLAY_TIME);

}

}