// Based on File > Examples > Servo > Knob

// Controlling a servo position using a potentiometer (variable resistor)

// by Michal Rinott <http://people.interaction-ivrea.it/m.rinott>

#include <Servo.h>

Servo myservo;  // create servo object to control a servo

int potpin = 0;  // analog pin used to connect the potentiometer

int val;    // variable to read the value from the analog pin

void setup()

{

Serial.begin(9600);

myservo.attach(9);  // attaches the servo on pin 9 to the servo object

}

void loop()

{

val = analogRead(potpin);            // reads the value of the potentiometer (value between 0 and 1023)

Serial.println(val);

val = map(val, 50, 300, 0, 179);     // scale it to use it with the servo (value between 0 and 180)

myservo.write(val);                  // sets the servo position according to the scaled value

delay(15);                           // waits for the servo to get there

}