/*     ---------------------------------------------------------

*     |  Experimentation Kit for Arduino Example Code         |

*     |  CIRC-14 .: Fancy Sensing :. (Soft Potentiometer)     |

*     ---------------------------------------------------------

*

*    Will fade an RGB LED from Red-Green-Blue in relation to the

*    soft pot value

*

*/

// LED leads connected to PWM pins

const int RED_LED_PIN = 9;    //Red LED Pin

const int GREEN_LED_PIN = 10; //Green LED Pin

const int BLUE_LED_PIN = 11;  //Blue LED Pin

void setup() {

//no need for any code here

}

void loop() {

int sensorValue = analogRead(0);   //read the Soft Pot

int redValue = constrain(map(sensorValue, 0, 512, 255, 0),0,255); //calculate the red Value (255-0 over the range 0-512)

int greenValue = constrain(map(sensorValue, 0, 512, 0, 255),0,255)-constrain(map(sensorValue, 512, 1023, 0, 255),0,255);  //calculate the green value (0-255 over 0-512 & 255-0 over 512-1023)

int blueValue = constrain(map(sensorValue, 512, 1023, 0, 255),0,255); //calculate the blue value 0-255 over 512-1023

// Display the requested color

analogWrite(RED_LED_PIN, redValue);

analogWrite(GREEN_LED_PIN, greenValue);

analogWrite(BLUE_LED_PIN, blueValue);

}