int motorPinL = 9;         //Arduino pin motor 1 is connected to

int motorPinR = 10;        //Arduino pin motor 2 is connected to

int steeringPin = 0;      //Arduino Analog pin 0 steering is connected to

int steerValue = 0;        //variable for steering

int hornButton = 3;

int hornPin = 2;

void setup(){              //happens once when Arduino starts up

pinMode(motorPinL, OUTPUT);  //declares motorPins as outputs

pinMode(motorPinR, OUTPUT);

pinMode(hornPin, OUTPUT);   //declares hornPin as output

}

void loop(){

getSteerage();     //calls getSteerage method

steerIt();                         //calls steerIt method

if (digitalRead(hornButton) == LOW){ //checks to see if horn is pushed

beep();                             //if horn is pushed calls "beep" method

}

}

void getSteerage(){             //read steering pin

steerValue = analogRead(steeringPin); //gives number between 0 and 1023

}

void steerIt(){

if (steerValue <= 255){         //turn left if steerValue is low

analogWrite(motorPinL, 255);

analogWrite(motorPinR, 0);

}

if (steerValue >= 256 && steerValue <= 767){

analogWrite(motorPinL, 255); // go straight if steerValue is in middle

analogWrite(motorPinR, 255);

}

if (steerValue >= 767){

analogWrite(motorPinL, 0); // turn right if steerValue is high

analogWrite(motorPinR, 255);

}

}

void beep(){                    //Makes piezo beep a "e" note

for(int i = 0; i < 100; i++){

digitalWrite(hornPin, HIGH);

delayMicroseconds(1519);      //1519 = "e" note

digitalWrite(hornPin, LOW);

delayMicroseconds(1519);

}

}